- COUNTY OF MONTEREY

- HEALTH DEPARTMENT

- ENVIRONMENTAL HEALTH BUREAU

# Cross-Connection Control for Community Water Systems

Purpose of Cross-Connection Control Program:  Water provided by a public water system may be contaminated via cross-connections within the user’s distribution system.  Section 7584 of Title 17, California Code of Regulations, requires that the water supplier protect the public water supply from contamination by eliminating actual cross-connections and reducing the hazard of potential cross-connections.  Cross-connections can be eliminated either by installing appropriate backflow prevention assemblies or ensuring that water-using equipment is installed in accordance with plumbing code requirements and good practice.

What are cross-connections?:  Cross-connections are actual and potential unprotected connections between a potable water system and any source or system containing unapproved water or a substance which is not safe.  Examples of cross-connections include:

1.	Improperly installed irrigation systems which may allow backsiphonage of stagnant, bacteriologically unsafe water into the domestic piping system.

2.	Improperly plumbed water-using devices such as hot tubs, boilers or commercial dishwashers, which may allow unsafe water back into the domestic piping system.

3.	Irrigation systems served by an auxiliary source, such as a private well or creek.  Such systems create a potential for major contamination of the public water system via inter-ties with the domestic piping system.

4.	Interconnections between the potable system and a non-potable system.

Why Cross-Connection Control Programs are necessary:  Cross connections involve several public health concerns:

- Consumers assume water from the tap is safe to drink.

- Contamination from cross connections can cause illness and death.

- Cross connections can contaminate even the highest quality water.

- Contamination may affect a single structure or may involve an entire community.

What the Regulations Require:  Section 7584 of Title 17, California Code of Regulations, requires that each public water system have a cross connection control program that includes these elements:

1.	The adoption of operating rules or ordinances to implement the cross-connection program.

2.	The conducting of surveys to identify water user premises where cross connections exist or are likely to occur.

3.	The provisions of backflow protection by the water user at all connections where a cross connection hazard has been identified.

4.	The provision of at least one person trained in cross connection control to carry out the program.

5.	The establishment of a procedure or system for testing backflow prevention assemblies.

6.	The maintenance of records of locations, tests, and repairs of backflow prevention assemblies within each water supplier’s distribution system.

Getting Started:   For community water systems, the initial elements of the program consist of the following:

1.	Adopting an ordinance or set of rules to implement the cross-connection control program.  The ordinance is important since it establishes the legal authority to carry out the program.

2.	Conducting a system survey to identify actual and potential cross-connection hazards.

3.	Ensuring that hazards identified by the survey are abated by installing backflow prevention assemblies at the meter, eliminating the hazard in conjunction with the owner of the property or providing internal cross-connection protection.

### System Survey

The system survey consists of a preliminary survey and, if necessary, a more detailed second survey. The survey shall be conducted by someone approved by the Department.  For most community systems, the initial survey may consist of a questionnaire sent to each customer asking whether the customer has specific potential hazards. Documentation of the system survey is to be submitted to the Department.  Attached is a summary form for documentation of the system survey.

Residential areas:   Customers should be asked if any of the following are located on-site:

1.	Auxiliary water supply (i.e. either a well, pond, spring, or a creek pump) - backflow prevention device is mandatory.

2.	Irrigation systems - backflow prevention device not required if system is installed in accordance with the Uniform Plumbing Code with appropriate vacuum breakers.

3.	Swimming pool, hot tub or spa - backflow prevention device not required if system is installed in accordance with the Uniform Plumbing Code.

4.	Solar hot water heating panels - backflow prevention device not required if system is installed in accordance with the Uniform Plumbing Code.

- Graywater systems - Backflow prevention assemblies may not be required if the system is installed in accordance with the Uniform Plumbing Code.

- Animal troughs – Filling mechanism (i.e., spigot with hose) may require backflow prevention device.

If these or other potential hazards are located on site, the water system shall determine whether the equipment has been installed in accordance with the Uniform Plumbing Code and good practice to minimize the risk of backflow occurring.

Commercial customers:  A more detailed questionnaire and survey is necessary.  Small community systems that also serve commercial customers should review the Department of Public Health Service’s “Manual of Cross-Connection Control - Procedures and Practices”, which can be found on our website.  A system survey of commercial users as specified in the Manual is to be performed.  As an alternative, the system may decide to require backflow prevention assemblies at all commercial service connections where hazards are likely to exist.

Wastewater and Hazardous Wastes:  Any service connections that handle wastewater or dangerous chemicals require special evaluation and protection from cross-connection hazards.  For additional information on evaluating these types of facilities, contact the appropriate regulatory agency and a cross-connection control specialist.

Attachment   - 	Summary Form for System Survey

##### Cross-Connection Survey Summary Form

Community Water Systems

Name of System _______________________________      System Number _______________________

#### Date of Survey_________________________________

Description of Survey Procedures - How survey was conducted (include copy of survey form):

Person conducting survey (List name and qualifications):   ______________________________________

_____________________________________________________________________________________

Procedures for Residential Connections:  ____________________________________________________

_____________________________________________________________________________________

__________________________________________________________________________________________________________________________________________________________________________

Procedures for Commercial Connections:____________________________________________________

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Total number of service connections ____________   Number of service connections surveyed  _________

Number of connections with auxiliary sources (i.e. wells, springs, or creek pumps)  ___________

Number of connections with other hazards 	____________

Total number of backflow prevention devices ____________

| Type of Hazard Identified (i.e. private well, hot tub, irrigation system, swimming pool, etc) | Number of connections with hazard | Number of devices installed | Number where device not necessary |
| --- | --- | --- | --- |

Describe follow-up for service connections which did not respond to the survey: _________________

_____________________________________________________________________________________

Long-term (Describe on-going cross-connection protection & testing of backflow prevention assemblies):

_____________________________________________________________________________________

__________________________________________________________________________________________________________________________________________________________________________

Submitted by (signature)______________________________________ Date ________________________